| PART NUMBER | COMPONENT NAME | USE | Operational voltage | Current draw | POWER CONSUMTION |
| --- | --- | --- | --- | --- | --- |
| HLK – 5M12 | AC – DC converter | Power supply | 12V | 450mA – 1A | 5 – 5.4W |
| L78M05ABDT - TR | 5V LDO | Fixed 5v output | 5V | 5mA | 25mW |
| LM1117DT -3.3 | 3.3V LDO | Fixed 3.3v output | 3.3V | 5mA | 25mW |
| ESP32 WROOM 2 | ESP32 | controller | 3.3V | 10mA – 500mA | 660mW |
| PC817 | optocoupler | Circuit isolation | 1.2V | 5-20mA | 6-24mW |
| BTA16 – 600B | triac | AC switching | 1.5 – 3V | 5 – 50mA | 7.5 – 150mW |
| ACS712Xlctr-20A | Current sensor | AC current sensing | 5v | 10mA | 50mW |
| TLV9001IDCK | Operational amp | AC voltage sensing | 1.8 – 5.5v | 0.06mA | 0.1 – 0.33mW |
| HCNR200-500E | Precision optocoupler | Isolation with precision analog voltage level balancing | 1.6V | 10-20mA | 16 – 32mW |
| WS2812B | Addressable LED | Status indicator | 5v | 0.3mA – 60mA | 1.5mW – 300mW |
| PMOS | mosfet |  |  |  |  |
| NPN | transistor |  |  |  |  |